Application Form

- Firm Name :- …………………………………………………………………………..

- Head Office Address :-………………………………………………………………...

Telephone:-………………………..  Fax………………………………

Email…………………………………………………………………..

- Responsible Person……………………………………………………………………..

Position…………………………………………………………………...

Telephone………………… Mobile………………. Fax:-…………….

Email:-…………………………………………………………………

- Firm Registration No…………………… RegisteredOffice………………………..

Date………………………………………………………………

- PAN/VAT Registration No………………………. Date………………………….......

- Business Type/ Purpose :…………………………………………………………….

- Financial Situation of Last three Years. ( Attach Audited report of three years)

Information from Balance sheet

| Fiscal Year | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Total Assets |  |  |  |
| Total Liabilities |  |  |  |
| Net Worth |  |  |  |
| Current Assets |  |  |  |
| Current Liabilities |  |  |  |

Information from Income Statement

| Fiscal Year | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Total Revenues |  |  |  |
| Profit Before Tax |  |  |  |
| Profit After Tax |  |  |  |

Financial Resources ( Add if required )

| No. | Source of Financing | Amount |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |

Note – The letter from the bank must be unconditional

- Human Resource Capability

| S.N. | Name | Position | Qualification ( Must submit certificate copy)* | Experience (Year) |
| --- | --- | --- | --- | --- |
|  |  | Manager |  |  |
|  |  | Account/Sales/ Marketing |  |  |
|  |  | Engineer/Sub Engg/Mechanics |  |  |

*Attach C.V. & certificate otherwise will not be considered for evaluation.

9.  Rate and details of the Equipment’s that the firm can supply as of annex-1 (Attach the    relevant catalogs, specifications etc.)

| S.N. | Name of the equipment | Estimated per unit cost | Brand | Model | PowerHP | Per unit cost ( Including VAT & Tax) |  | Free Spare Parts including Tool Box | No. of Free servicing after sales |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Rate in figures | Rate in words |  |  |
| 1 | Cultivator 9 tynes | 45000 |  |  |  |  |  |  |  |
| 2 | Rotavator 6ft 42 blades | 185000 |  |  |  |  |  |  |  |
| 3 | Land Leveller single/Reversible | 24000 |  |  |  |  |  |  |  |
| 4 | Disc Harrow bush/bearing type | 95000 |  |  |  |  |  |  |  |
| 5 | Disc plough round | 75000 |  |  |  |  |  |  |  |
| 6 | Tractor operated Wheat Thresher | 385000 |  |  |  |  |  |  |  |
| 7 | Tractor Operated Paddy Thresher | 385000 |  |  |  |  |  |  |  |
| 8 | Tractor operated Maize Thresher (Conveyer Model) | 385000 |  |  |  |  |  |  |  |
| 9 | Tractor operated Maize Thresher | 340000 |  |  |  |  |  |  |  |
| 10 | M.B. plough 2 bottom | 42000 |  |  |  |  |  |  |  |
| 11 | Sub soiler 2 tynes/chisel plough | 65000 |  |  |  |  |  |  |  |
| 12 | Mini Tiller 170 f petrol engine | 55000 |  |  |  |  |  |  |  |
| 13 | Mini Tiller 178 f Disel engine | 85000 |  |  |  |  |  |  |  |
| 14 | power tiller DF20 | 250000 |  |  |  |  |  |  |  |
| 15 | power sprayer petrol engine | 45000 |  |  |  |  |  |  |  |
| 16 | power weeder | 40000 |  |  |  |  |  |  |  |
| 17 | Self -propelled reaper | 195000 |  |  |  |  |  |  |  |
| 18 | Sugarcane planter 2 row | 520000 |  |  |  |  |  |  |  |
| 19 | Double Trench Ridger | 75000 |  |  |  |  |  |  |  |
| 20 | Manual sugarcane bud Cutter | 7000 |  |  |  |  |  |  |  |
| 21 | inter row Rotary weeder 3 row | 230000 |  |  |  |  |  |  |  |
| 22 | Hydraulic trolly | 350000 |  |  |  |  |  |  |  |
| 23 | Tractor 50 HP category | 1576350 |  |  |  |  |  |  |  |

Note:- If the proposed amount of the Agri-equipments is greater than  estimated per unit cost  then the firm/company who has been proposed that amount will be eliminated.

10. Availability of Equipments that the firm can supply (For this FY)

| S.N. | Name of the equipments | Brand | Model | Available in stock (N0s) | Quantity that can be supplied in this FY (NOs) | Rate in figures | Total Amount (NRs)* |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Cultivator 9 tynes |  |  |  |  |  |  |
| 2 | Rotavator 6ft 42 blades |  |  |  |  |  |  |
| 3 | Land Leveller single/Reversible |  |  |  |  |  |  |
| 4 | Disc Harrow bush/bearing type |  |  |  |  |  |  |
| 5 | Disc plough round |  |  |  |  |  |  |
| 6 | Tractor operated Wheat Thresher |  |  |  |  |  |  |
| 7 | Tractor Operated Paddy Thresher |  |  |  |  |  |  |
| 8 | Tractor operated Maize Thresher (Conveyer Model) |  |  |  |  |  |  |
| 9 | Tractor operated Maize Thresher |  |  |  |  |  |  |
| 10 | M.B. plough 2 bottom |  |  |  |  |  |  |
| 11 | Sub soiler 2 tynes/chisel plough |  |  |  |  |  |  |
| 12 | Mini Tiller 170 f petrol engine |  |  |  |  |  |  |
| 13 | Mini Tiller 178 f Disel engine |  |  |  |  |  |  |
| 14 | power tiller DF20 |  |  |  |  |  |  |
| 15 | power sprayer petrol engine |  |  |  |  |  |  |
| 16 | power weeder |  |  |  |  |  |  |
| 17 | Self -propelled reaper |  |  |  |  |  |  |
| 18 | Sugarcane planter 2 row |  |  |  |  |  |  |
| 19 | Double Trench Ridger |  |  |  |  |  |  |
| 20 | Manual sugarcane bud Cutter |  |  |  |  |  |  |
| 21 | inter row Rotary weeder 3 row |  |  |  |  |  |  |
| 22 | Hydraulic trolly |  |  |  |  |  |  |
| 23 | Tractor 50 HP category |  |  |  |  |  |  |

*Please submit deposit voucher or bank guarantee of 5% of the Total Amount (NRs.) that the firm can supply in current FY. If not submitted the firm will not be eligible for evaluation.

11. Availabilities of Spare Parts (Attach separate sheet if required)

| S.N. | Parts Name | Quantity | Unit Rate in figures | Amount |
| --- | --- | --- | --- | --- |

Total Spare Parts available in NRs…………………………….( must be quoted for evaluation purpose)

12. Dealer, Branches and Distributes in the country

| S.No. | Name | Address | Contact person | Email | Telephone | Mobile |
| --- | --- | --- | --- | --- | --- | --- |

Submit copy of dealership agreement with firm registration, PAN or VAT registration otherwise will not be considered for evaluation.

13.  Service Centers

| S.N. | Name | District | Types of services Available ( Routing/Regular/Engine Overhaul) | Address | Contact Person | Email | Telephone | Mobile |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Submit copy of dealership agreement with firm registration, PAN or VAT registration otherwise will not be considered for evaluation.

14. Professional Experiences

15. Specific Experience (Please submit supporting documents)

I hereby declare that the above submitted information are true and correct based on relevant documents and our knowledge and we are not ineligible to participate in the expression of interest, has no conflict of interest in the proposed procurement proceedings and has not been punished for the profession or business related offence.

Office   stamp

(Signature of Authorized Person)

Name ………………………….

Position ………………………..

Technical specification of Agricultural equipment

Office: Agriculture Knowledge centre, Kalaiya (Bara)	 F/Y: 2082/83

| S. N | Required Technical Specifications |  | Bidder's specification |
| --- | --- | --- | --- |
|  | Cultivator |  |  |
|  | Tractor operated spring-loaded cultivator. No of Tynes:9 Tines or more; Type: spring Type (18 coils). Tractor Power: 36 hp & above. working width:80”, Under frame clearance 15 inches, Weight: 210 kg. approx. frame (50x6 mm). Type of mounting: Three points linkage |  |  |
|  | Rotavator |  |  |
|  | Overall length dimension LXBH: Not less than1800X800X1030 mm approx., Working width 1670 mm. Three points hitch CAT-II. Multi speed gear driven, minimum 42 no. of blades with 7or more flanges, BLADE TYPE STANDARD (Curved). Gear drive side transmission, rotor shat tube diameter 75 mm approx. and minimum weight 350 kg. Driveline Safety Device: Shear Bolt / Slip Clutch. |  |  |
|  | Leveller |  |  |
|  | Approximate 6 ft long; Suitable for all kinds of levelling work at field and sites; Can be attached with 35 to 60 H. P. Tractors; Large capacity of carrying clay; >5 mm main plate thickness, Main plate length about 1600 mm, Side support >5 mm thickness Carbon steel blade. Weight about 90-100 kg. |  |  |
|  | OFFSET Disc harrow |  |  |
|  | Tractor operated disc harrow. No of discs: 14 or more; type of disc: front gang notched and rear plain. Disc diameter 18-inch x 4 mm. width of cut 48”or more. Tractor Power required: 40 hp & above. Weight 350 kg approx. Frame: std. channel and box made angle, gang bolt std. Type of mounting: Three points linkage. |  |  |
|  | Disk plough |  |  |
|  | Directly mounted to the Tractors. hydraulically operated 2or 3 Disc version. Weight Approx: 200 Kg. Max Working Width: 29” |  |  |
|  | Tractor operated Wheat Thresher |  |  |
|  | General | Tractor P.T.O operated three fan Wheat thresher |  |
|  | Capacity & characteristics | Above 900 kg per hour/hr std condition efficiency 97% or above, overall grain loss% less than 2.5, broken grain loss less than 0. 8%, blown grain less than 1.5% |  |
|  | Matching power source | 35 hp and above Tractor |  |
|  | Crop Feeding Mode | Conveyor type with two feed roll, belt width more than 240mm and belt thickness more than 3mm.or chute type |  |
|  | Feeding hopper (±2%) | Std. |  |
|  | Threshing cylinder | Cylindrical Rotor type, flats/Hammer/rasp bar/ wire loop/Pegs 24 minimum in number bolted square bar, diameter of drum more than 400mm and width 600 mm, Clearance between threshing drum and half round concave 15-18 mm; Turning speed of threshing cylinder shaft not less than 400 RPM, Main Shaft Diameter Not more than 80 mm |  |
|  | Sieves | Oscillating perforated sieve, Minimum 2 number |  |
|  | concave | Half round made of sq. bar |  |
|  | Cleaning fan | Grain cleaning Aspiratory type fan 2 number and straw blower of centrifugal type |  |
|  | Overall Dimension (±5%) | Std. |  |
|  | Hopper height from Platform | Not less than 500 mm. |  |
|  | Weight (Approx.) | Above 1050 Kg |  |
|  | Re cleaning mechanism | Standard |  |
|  | Feeding platform | Standard, |  |
|  | Hitching, transport wheels, frames | Standard, pipe with height adjustment, TWO wheels. |  |
|  | Safety cover | Standard |  |
|  | Standard Accessories | As Per Need |  |
|  | MS Sheet Thickness | Min 2 mm (Covered Part) |  |
|  | Test report | Authorized test report |  |
|  | Tractor operated Paddy Thresher |  |  |
|  | General | Tractor P.T.O operated paddy thresher |  |
|  | Capacity & characteristics | Above 1200 kg/hr., threshing efficiency 98%, cleaning 97%, overall grain loss% less than 2.5, broken grain loss less than 0. 8%, blown grain less than 1.5% |  |
|  | Matching power source | 35 hp and above Tractor |  |
|  | Crop Feeding Mode | Chute type |  |
|  | Feeding hopper (±2%) | Std. |  |
|  | Threshing cylinder | Cylindrical Rotor type (Pegs /beater/drum/or equivalent in number bolted on U shaped channel, diameter of drum Not more than 1.0m, length min 1200mm, cover of MS SHEET 2.5mm) |  |
|  | Clearance between threshing drum and half round concave | concave made up of MS square bar 8-10mm gap |  |
|  | Turning speed of threshing cylinder shaft | Not more than 1000 RPM |  |
|  | Sieves | Oscillating perforated sieve, Minimum 2 number, Std. |  |
|  | Size of holes in sieves | 10-12mm (upper), 1.5-2 mm (lower) |  |
|  | Cleaning fan | Grain cleaning Aspiratory type fan two number and side blower one number of size std. grain cleaning fan of aspiratory type. |  |
|  | Overall Dimension (±5%) | Std. |  |
|  | Hopper height | Around 800 mm. from Platform |  |
|  | Weight (Approx.) | Not more than 2000 Kg |  |
|  | Re cleaning mechanism | Standard |  |
|  | Feeding platform | Standard |  |
|  | Hitching, transportation wheels, frames | Standard |  |
|  | Safety cover | Standard |  |
|  | Standard Accessories | As Per Need |  |
|  | MS Sheet Thickness (Curved Part) | More than 2 mm |  |
|  | Main Shaft Diameter | Std. |  |
|  | No. of Transmission Joints | More than 2 |  |
|  | flywheel | 2 no. diameter min 500mm |  |
|  | Test report | Authorized test report |  |
|  | Tractor operated Maize Thresher (conveyor + Elevator model) |  |  |
|  | General | Tractor P.T.O (540 rpm) operated maize DE husker cum sheller having conveyor for loading and elevator for grain discharge/loading, capable of removing husk/silk and separate cobs and grains. |  |
|  | Capacity & characteristics | Above 1500 kg per hour, threshing efficiency 95%. |  |
|  | Matching power source | 35 hp and above Tractor |  |
|  | Crop Feeding and grain lifting/discharge Mode | Conveyor feed/lifting and grain lifting by elevator (min 4 feet) loading /discharge mechanism. |  |
|  | Threshing cylinder | Threshing drum Dia: 50 inches, Angle frame: std. height 62 cm, shaft length 31; drum cover sheet 2 mm threshing cylinder shaft not less than 500 rpm |  |
|  | Sieves | Oscillating perforated sieve |  |
|  | concave | Half round |  |
|  | Weight (Approx.) | 1000 Kg around |  |
|  | Hitching, transport wheels, frames | Standard, pipe with height adjustment, TWO wheels. |  |
|  | Safety cover | Standard |  |
|  | Standard Accessories | As Per Need |  |
|  | MS Sheet Thickness | Min 2 mm (Covered Part) |  |
|  | Tractor operated Maize Thresher |  |  |
|  | General | Tractor P.T.O (540 rpm around) operated maize DE husker cum sheller capable of removing husk/silk and separate cobs and grains. |  |
|  | Capacity & characteristics | Above 1500 kg per hour, threshing efficiency 95%. |  |
|  | Matching power source | 35 hp and above Tractor |  |
|  | Crop Feeding and discharge Mode | Crop feeding Manual |  |
|  | Threshing cylinder | Threshing drum Dia: 52inches, length 105 cm, height 52 cm, shaft length 51; drum cover sheet 2mm threshing cylinder shaft std. rpm |  |
|  | Sieves | Oscillating perforated sieve |  |
|  | concave | Half round |  |
|  | Weight (Approx.) | 900 Kg |  |
|  | Hitching, transport wheels, frames | Standard. pipe with height adjustment, TWO wheels. |  |
|  | Safety cover | Standard |  |
|  | Standard Accessories | As Per Need |  |
|  | MS Sheet Thickness | Min 2 mm (Covered Part) |  |
|  | M.B Plough |  |  |
|  | Two bottom M.B or more. Plough pulled by 35 or above tractor. Three-point hitch, width of cut 610mm minimum, depth of cut 300mm, weight 220 kg approx. Tyne 34-36 mm, board 8 mm thick, length and height 1500X800x1040 mm. |  |  |
| 11 | Subsoiler/Chisel plough |  |  |
|  | chisel Plough or sub soiler pulled by 45 or above tractor. Three-point hitch, width of cut 600mm minimum, depth of cut 350mm, weight 200 kg approx. Number of Tines (2), Thickness of tine, mm (25). |  |  |
| 12 | Petrol Mini Tiller medium power category |  |  |
|  | Make & model | ISO certified, Minitiller | Mini tiller model: |
|  | Engine type and model | 4 stroke, single cylinder, air cooled gasoline engine. power: 4 kw or more | Engine Model no.: |
|  | Transmission system | wet type, friction Double clutch, no. of gear speed:2 forward &1 reverse and neutral |  |
|  | Model of transmission | driven by gear system |  |
|  | Number of rotor blades | 24 blades |  |
|  | Transportation wheel | Solid or Pneumatic tyre size (inch):4.0:8 |  |
|  | Starting method | recoil starting system |  |
|  | Goss Weight (Kg) | Not less than 80 | G.W. : |
|  | Width of cut (cm) | 75 to 100 |  |
|  | Depth of cut (cm) | 10 to 15 |  |
|  | Productivity (m2/h) | 600 to750 |  |
|  | Fuel tank capacity(liter) | 3.6 to 4 |  |
|  | Accessories parts & tools | One set of complete tool box, mini tiller filled with lubricant and Gear oil (Ready to use). |  |
|  | Servicing | Two free servicing. |  |
|  | Test reports | Compulsory Authentic test reports of engines |  |
| 13 | Diesel Mini Tiller medium power category |  |  |
|  | Make & model | ISO certified, Minitiller |  |
|  | Engine | 4 stroke, single cylinder, forced air cooled, Diesel engine. Power: 4 kw or more |  |
|  | Transmission system | friction wet type, Double clutch, no. of gear speed:2 forward &1 reverse and neutral |  |
|  | Model of transmission | driven by gear system |  |
|  | Number of rotor blades | 32 blades. |  |
|  | Transportation wheel | Solid or Pneumatic tyre size (inch): 4.0:8 |  |
|  | Starting method | Recoil starting system |  |
|  | Steering handle | Height adjustment provided. |  |
|  | Goss Weight (Kg) | Not less than 110 (excluding packaging box but including fuel/oil) |  |
|  | Width of cut(cm) | 105 to 120 |  |
|  | Depth of cut(cm) | 10-15 |  |
|  | Fuel tank capacity(liter) | 3.5 to 4 |  |
|  | Lubrication oil capacity (L) | 1 or more |  |
|  | Air cleaner | Oil bath type |  |
|  | Manual and documents | one set of operation manual. Test report or any documents to verify the power of mini tiller. related mini tiller’s model should be stated on test report. |  |
|  | Accessories parts & tools | One set of tool box, MT filled with lubricant & Gear oil (Ready to use). |  |
|  | Servicing | Two free servicing. |  |
| 14 | Power tiller (20 Hp Category) |  |  |
|  | General | Diesel Power tiller suitable for Terai land condition of Nepal. The weight of the power tiller with Rotavator shall be 400-450 kgs. |  |
|  | Engine | Horizontal single cylinder, 4 stoke Water Cooled diesel engine, The maximum power output shall be in the range of 20± (5%) HP at governed RPM. |  |
|  | Starting System | Hand Cranking |  |
|  | Transmission | Gear Type |  |
|  | Fuel Tank | Minimum 10 Liter |  |
|  | Cooling | Water radiator |  |
|  | Turning speed of rototilling shaft | 300-550 RPM |  |
|  | Rototilling width | 800 mm minimum, number of blades minimum 24 |  |
|  | Tyre size | 6.00-12 |  |
|  | Minimum ground clearance | 185 mm |  |
|  | Manual | One copy of the operator’s |  |
|  | Tools | A set of tools for general maintenance shall be supplied |  |
|  | Warranty | Manufacturer shall provide minimum warranty valid for one year after the date of acceptance. |  |
|  | Initial Service | Power tiller with all the necessary lubricants in ready to start condition |  |
| 15 | Power sprayer |  |  |
|  | Trolley power sprayer Suitable for spraying chemicals at farms, gardens, and agriculture land, 4 stroke, gasoline engine 0.7kw or more, Pressure ≥20 kg/sq.cm., hose pipe 20 m or more, output 18 liter or more per minute. Vertical spraying range 10 m. |  |  |
| 16 | Power weeder |  |  |
|  | Gasoline cultivator 4 stroke, single cylinder, air cooled gasoline engine. power: 1.5 kw or more. Starting method manual or recoil starting system. Fuel tank capacity(liter): 1.4L or more. Engine oil 0.35 liter. Handle Adjustment:3 Positions. Transport Wheel; Drag Bar Adjustable Height3 Positions. Dry Weight with Rotor 30kg or above. Accessories parts and tools: one set of complete tool box, cultivator filled with lubricant and Gear oil. |  |  |
| 17 | Self -propelled reaper |  |  |
|  | Self-propelled vertical conveyer Reaper Suitable for Harvesting of Wheat, Paddy, Mustard etc. Engine: Single cylinder, 4 stroke water cooled radiator type diesel engine. Power Requirements 6 (hp). Rated Engine Speed (rpm)3600, Engine Oil Capacity 1 (L), Minimum Fuel Tank Capacity (L)3.6 or above. Starting System: Hand Cranking; Mode of Transmission Through Belt and Pulley. Main Transmission Box-Chain and Sprocket. No. of Gears-Forward 1, Reverse 1. Tyres-5.00*12-8PR Pneumatic tyres. Steering System-Manual Clutch system. Productivity (m2/hr)-1800 or above; Minimum cutting height-50 mm. Minimum Cutting Width (mm)1200. No. of Crop Divider-5; No. of Guiding Wheels-4. Crop Conveyer-2 nos. Sidewise windrowing. G. Weight (Kg) 250 approx. |  |  |
| 18 | Sugarcane planter |  |  |
|  | Two row sugarcane planter drawn by tractor. Trench opening, seed cutting, planting, fertilizer application and covering simultaneously. Three-point linkage hitching. Seed tank: 600-700 kg, fertilizer tank: 70 kg. Furrow distance: adjustable, seed to seed: gap/overlap; number of blades 6, insecticide/fungicide tank100-200-liter, depth control: hydraulic and rubber wheel, number of seats 2, field capacity: 3 Acre/hr. Weight: 800 kg, Dimension: 2100x2100x2400 mm. |  |  |
| 19 | Row trench opener/ridger |  |  |
|  | Two Row Trench Planter opener/ridger for sugarcane planting, pulled by 35 or above tractor. Three-point hitch, working width:2500mm. Weight: 200-230kg; Semi-Automatic; Hydraulic Driven Type, Ground Clearance:400-450mm, depth of furrow about 12 inches. Heavy duty shovel, adjustable furrow size, adjustable row distance. Box Frame:65x6 mm; Tyne (4 nos). |  |  |
| 20 | Bud cutter |  |  |
|  | Manual sugarcane Bud cutter made up of MS, Weight approx. 3.5 kg. Attachment:1 Blade; Bud size (1.5 Inch or 3 Inch), can be made cutting size suitable to fit in seedling trays. Blades are grinded from both sides (top and bottom). Powder coated. |  |  |
| 21 | Inter row rotary tiller (3 row) |  |  |
|  | PTO operated Inter row rotary tiller power required to operate 45-60hp tractor. weight 600kg approx. working width 24-30cm” for each, rotor blades: L shape Blade thickness 6mm. depth of cut 5-10 cm. Transmission: primary and secondary gear. Depth control on each side. Three-point hitch mechanism. Material steel. |  |  |
| 22 | Hydraulic trolly |  |  |
|  | Size of trolly dimension: approx..3000*1800*525 mm. Floor sheet4 mm. Housing: Truck housing. Tyre:2 nos. Tyre size:9.0-20. Hydraulic capacity:15 ton. Side wall sheet 3.00 mm. Chasis:6"*3.5" approx.. |  |  |
|  | Delivery Place at site specified by office |  |  |

Technical Specification of Tractor (2WD 50 HP) Category

| S. N | Required Technical Specifications |  | Bidder's specification |
| --- | --- | --- | --- |
| 23 | Tractor (2WD 50 HP) Category |  |  |
|  | General | Model of tractor | Model: |
|  | Engine HP | 50 +3% @2200RPM |  |
|  | Number of cylinders | Not less than 3 |  |
|  | Engine Capacity | 2700 cc or above |  |
|  | Engine Type | Four stroke, Direct Injection, water-cooled, dry or oil bath type air cleaner/filter, diesel engine |  |
|  | Fuel Tank Capacity (lit) | More than 55 |  |
|  | Electrical System | 12V/75Ah or more Battery, 12V 36A Alternator, Self Starter Motor/ 12V, 2.2 KW r, Head lamps, Fender & Tail Lamps |  |
|  | Steering Type | Power Steering |  |
|  | Clutch | Dual Type |  |
|  | Transmission |  |  |
|  | Number of Gears | At least 8 Forward, at least 2 Reverse |  |
|  | Type | Synchromesh OR Sliding mesh |  |
|  | Transmission Type | 2WD |  |
|  | Brakes Type | Oil Immersed DISC Brakes |  |
|  | Hydraulic System |  |  |
|  | Pump (type) | Open centre and draft control |  |
|  | Hitch lift capacity | At least 1800 kg |  |
|  | 3-Point Linkage | Automatic depth and Draft control (ADDC). Links Fitted with Cat 1 And Cat 2 Balls (Combi Ball) |  |
|  | PTO (RPM) | Six splined shafts (540+10%) |  |
|  | PTO Speed Selection Method | Lever |  |
|  | Wheel base | 1900-2100 mm |  |
|  | Ground clearance | Not less than 400 mm |  |
|  | Tyres (Front/Rear) | Standard or 6×16” 8PR (F), 14.9×28” 12PR (R) |  |
|  | Tools Set | Available |  |
|  | Top link | Available |  |
|  | Canopy (Hood) | Optional |  |
|  | Hooks | Available |  |
|  | Drawbar | Available |  |
|  | Front Weight | Available |  |
|  | Warranty | 2 Years |  |
|  | Test report | Authorized test report |  |
|  | Servicing | Two free servicing. |  |
|  | Delivery place | At site specified by office |  |

Signature of Supplier………………….

Date……………………………

Office Stamp